# ImRotate GUI user manual

# System Set-Up

- Connect DE2 board power supply.

- Connect DE2 to external display using VGA port.

- Connect DE2 RS-232 port to PC. Make sure that it connects to port COM4 on PC (else change Matlab GUI code to appropriate port ).

- Make sure RS-232 is defined to baudrate 115200.

- Connect JTAG to PC.

- Power up DE2 Board0

- Burn the system File "img_man_MDS.sof" to the DE2 card using Quartus Programmer.

# Operation Sequence

- Start Matlab and run ImRotateGUI.m.

- Load images using Load Image button. image should by 512x512 8bit greyscale image(bmp/jpeg). Loaded images should in a list below.

- Choose image from list of loaded images.

- Choose manipulation parameters:

  - Rotation angle – 0 up to 360 degrees.

  - Zoom Factor – integer 1 up-to 16.

  - Vertical/Horizontal Start Point– between 1 and 512 (according to input image resolution).

- Press Transmit Image.

6

4

3

5

2

- If Synthetic Image option is selected, image will not be displayed. A synthetic generated image will be displayed

# Extra Features

Switch 0 –

Down -manual image switching is disabled.

Up – manual image switching enabled.

Switch 1 –

Down –display manipulated image.

Up – display original image.

KEY0

Reset when pressed